European Union Rule of Law Mission in Kosovo

EULEX Kosovo

Ndertesa Farmed

“Muharrem Fejza” p.n.

Lagja e Spitalit

10000 Pristina, Kosovo

www.eulex-kosovo.eu

- CORRIGENDUM (3)

PUBLICATION REFERENCE: PROC/361/12/Residential Security Upgrade Kosovo wide FWC (EuropeAid/133523/L/SUP/XK)

- The following corrections are made to the Annex II+III Technical Specifications:

The technical and financial evaluation of the offers will be made on the basis of these amended Technical Specifications.

| Item No | Former text of the technical specifications | Shall read as new text: (added/amended text in red) |
| --- | --- | --- |
|  | Security Doors - Steel metal frame of 60mm x 40mm width. - Steel plate 2x2 mm each sides. - Security viewing hole - Dead bolt lock. - Handle - Minimum 2 Security locks. Cylinders must meet at least European norm EN 1303 Grade 4. To be equipped with a special protection to prevent wrenching from the outside, of a drill-resistant plate and also to be protected from burglar tools - Minimum 3 keys per lock. - Compliant to ENV 1627 Class 5, tested according to ENV 1630. - Security hinges must be of the anti-rise type must be installed on the inside of the doors (not accessible from outside). | Security Doors - Compliant to ENV 1627 Class 3, tested according to ENV 1630 - Steel metal frame of 60mm x 40mm width (+/-10mm) - Rigid steel sub-frame. - Door thickness: minimum 50mm - Double-sided whole-area armoring with 1-1.5 mm steel sheet. - Armoring reinforced with minimum 4 mm hardened steel sheet in the area of the lock - Stainless steel hinges with security dog bolts for security. - Security viewing hole - Dead bolt lock. - Handle. - Security door lock must meet at least European norm EN 12209 Grade 3. To be equipped with a special protection to prevent wrenching from the outside, of a drill-resistant plate and also to be protected from burglar tools - Minimum 13 security points. - Minimum 3 keys per lock. - Door size: 90cm (width) x 200cm (length) (wing dimension). Surfaces finish: veneered planks or solid wood panels. |
| 1.1 | Installation of Security Doors Installation of Security Doors – GARDESA with following technical specifications: - Steel metal frame of 60mm x 40mm width. - Steel plate 2x2 mm each sides. - Security viewing hole - Dead bolt lock. - Handle - Minimum 2 Security locks. Cylinders meet European norm EN 1303 Grade 4. Equipped with a special protection to prevent wrenching from the outside, of a drill-resistant plate and also to be protected from burglar tools - Minimum 3 keys per lock. - Compliant to ENV 1627 Class 5, tested according to ENV 1630. Note: EULEX will provide the Security doors therefore for this item we require only the installation services. | Installation of Security Doors Installation of Security Doors – GARDESA with following technical specifications: - Steel metal frame. - Steel leaf 18/10mm. - Security viewing hole - Dead bolt lock. - Handle - Minimum 3 keys per lock. - Door size: min.85cm (width) x max.210cm (length) (wing dimension) - Compliant to ENV 1627 Class 3, tested according to ENV 1630. Note: EULEX will provide the Security doors therefore for this item we require only the installation services. |