EULEX KOSOVO

TECHNICAL SPECIFICATIONS

Building Management Unit

EULEX Kosovo

May 2026, Pristina

1	GENERAL INFORMATION	3

2	SECURITY CONDITIONS	3

3	SCOPE OF WORKS	3

3.1	CONTRACTOR’S RESPONSIBILITIES	4

3.2	TECHNICAL SUBMISSION AND TIME SCALE	4

3.3	PRIOR TO COMMENCING OF ANY WORK	4

3.4	Final Clearance	5

3.5	WHEN THE CONTRACTOR HAS OBTAINED APPROVAL FROM EULEX	5

3.6	GENERAL WORK REQUIREMENTS	5

x)	A short guide to the personal protective equipment at work (PPE)	8

4	DESCRIPTION OF MATERIALS AND WORKS	13

4.1	Dismantling of Containerized Blocks in Prishtina	13

4.1.1	Dismantling of Container Block #3	13

4.1.2	Dismantling of Container Block 4	15

4.1.3	Dismantling of Container Block 5	17

4.1.4	Dismantling of partition walls and floor layers (standalone containers in ESC)	18

4.1.5	Refurbishment of steel structure for all office and ablution containers	19

4.1.6	Transportation of containers.	20

4.2	Dismantling of IFM Block	20

4.3	Dismantling of Container Blocks in Mitrovica	21

4.3.1	Dismantling of Reception #01	21

4.3.2	Dismantling of Block # 02	22

4.3.3	Dismantling of Block K	23

4.3.4	Dismantling of Infirmary #04	24

4.3.5	Treatment of Steel Structure for all containers and ablution units	25

4.3.6	Transportation of containers	25

# GENERAL INFORMATION

EULEX in this project has foreseen dismantling of container blocks in EULEX Operations and Support Compound in Prishtina, Container Block located within the yard of Institute of Forensic Medicine in Prishtina and in EULEX Compound in Mitrovica.

Project include dismantling works, removal of debris and recovering of containers structure for further use.

Blocks are located:

- Industrial Zone, Street “Lidhja e Pejes”, Prishtina

- QKUK Prishtina

- Ferki Lisica Street, Mitrovica

# SECURITY CONDITIONS

Security Conditions that have to be considered for the performing companies:

- Initially a list of workers and their machinery that they will use during their work will be asked from the contractor.

- The appointed supervisor will meet the contractor’s representative before they start with works and they will consider the security procedures.

- Contractor’s tools which are not going to be used during the weekend will be asked to be securely stored on site (except if they have no request to work during the weekends).

- All the tools shall be locked in tool boxes when not in use (during weekdays and nights).

- The supervisor will check all the working places before the works start and after the works are finished for that particular day (every day).

- No items shall be transported in or out of the site without the express prior approval of the designated EULEX supervisor. The approval, preceded by an inspection of the items to be transported, shall be in writing and will be accompanied by the list of the items approved for transportation to the specific location.

# SCOPE OF WORKS

Works include:

- Location: EULEX Operations and Support Compound in Prishtina:

EEPC:

- Dismantling of Containerized Block 3

- Dismantling of Containerized Block 4

- Dismantling of Containerized Block 5

- Dismantling of partition walls and floor layers – standalone containers in ESC.

- Refurbishment of containers steel structure

- Transportation of Containers from Prishtina to Mitrovica.

- Dismantling of Container Block in IFM – Prishtina

- EULEX Compound Mitrovica:

- Dismantling of Block 1

- Dismantling of Block 2

- Dismantling of Block K

- Dismantling of Infirmary #4

- Refurbishment of containers steel structure

- Transportation of containers within Mitrovica.

## CONTRACTOR’S RESPONSIBILITIES

Tenderers will be given the opportunity to visit the site and familiarize themselves with existing conditions. All tenders will assume to have satisfied themselves concerning all relevant details pertaining to the contract. Site visit is mandatory. Tenders that have not attended site visit will not be taken into consideration for application.

## TECHNICAL SUBMISSION AND TIME SCALE

Tenders are obliged to include sufficient technical information to allow EULEX to assess the quality of proposal. Tenders are obliged to state the contract period and the warranty period in their submission and submit a schedule of works in the form of bar chart with regard to the work that needs to be carried out on the site. This information will be taken in to account when proposals are assessed and evaluated.

## PRIOR TO COMMENCING OF ANY WORK

The contractor shall provide the samples of all materials requested by EULEX representative with the manufacturer specifications that he intends to use, for approval by EULEX designated representative.

The contractor shall submit a complete program which specifies his intended procedure for executing the work in compliance with the above-mentioned requirements.

The contractor shall take his own measurements in accordance with the physical status on the ground before starting of any productions.

## Final Clearance

Upon completion of each section of the works, the Contractor shall clean up the site; remove all temporary buildings, plant if any, and remove debris. He shall level off and fine grade all excavated materials which is surplus to requirements. The whole of the site shall be left in a clean and workmanlike condition to the satisfaction of the Supervisor.

## WHEN THE CONTRACTOR HAS OBTAINED APPROVAL FROM EULEX

All work has to be carried out to the complete satisfaction of the delegated representative in accordance with relevant standards and good working practices.

The Contractor shall notify the designated representative, prior to covering any part of the work, which will then be inaccessible for subsequent inspection. Failure to do so will result in exposure of the afore-said works and subsequently reinstatement at the contractor’s expense.

It is essential that contractor has an English speaking supervisor with direct authority over the workforce on site during all working hours. In the absence of an English-speaking supervisor, a translator will be required to be present at all times.

## GENERAL WORK REQUIREMENTS

#### Security and Fire Fighting

The Contractor shall provide and maintain adequate fire extinguishers on the site and areas of high fire risk shall be fenced and signs posted and supplied with specialized fire extinguishers, if necessary. Generators and their batteries and water pumps shall be adequately protected against vandalism and theft. Unless otherwise provided by the Supervisor, the Contractor shall not by his operations obstruct any road or access to other buildings nor break down any fence nor obstruct any drains or water courses, but if such blockages occur he shall at once remove the blockages and repair the breakages.

#### Construction Equipment & Machinery

The Contractor shall provide all necessary sound and adequate equipment, heavy machinery and vehicles machinery vehicles and equipment required for the works that include but are not limited to (cranes, forklifts, trucks etc). The Contractor shall be solely responsible for the employment of skilled and qualified machinery and equipment operators as well as the sufficient and proper maintenance of all machinery and equipment employed on the site.

The Contractor shall be solely responsible for the adequate lighting where work is being executed at night and shall provide and install any additional lighting which the Supervisor may require in order to watch and supervise the works and carry any testing and examination of materials. Materials available on the site or materials made available or supplied by the Contractor shall be used solely for the execution of the works. The Contractor shall minimize the pollution of and disturbance to roads and other places on and around the site. No trees or other vegetation shall be removed except with the express permission of the Supervisor. The Contractor shall ensure that access is provided to all buildings and properties adjacent to the site for the duration of the contract.

All temporary buildings erected by the Contractor upon the sites and the layout of the buildings and the site, shall comply with laws and all local by-laws in so far as they are applicable. The Contractor shall be absolutely and solely responsible for the safety and security of temporary works and for the equipment in connection therewith which may be erected or provided for the carrying out of the contract and for the execution of the works. This provision shall be applicable to all temporary works and equipment whenever provided and erected by the Contractors for the purpose of or in connection with the works. Examination and acceptance by the Supervisor of the Contractor’s temporary works or of the drawings connected therewith shall not absolve the Contractor from his responsibility for those works and his liability for the consequences of any failure.

#### Water Supply

EULEX will allow use of water within the Compound during the construction works; however connection points are to be previously authorized by the Contracting Authority. Water consumptions during the works are not to be charged. Contractor is to undertake all arrangements if provision of pumps, storage tanks etc as required.

#### Electric Power Supply

EULEX will allow the Contractor the connection of electricity during their works to a specific connection points. Power consumption during the works is not to be charged. However the Contractor is to provide all wiring, lamps, switches, fuses, receptacles, etc., as may be required for his work. Temporary power and light circuits are to be thoroughly insulated and waterproof. The temporary power and light system are to be subject to the inspection and approval of the appropriate authority. All the used electrical materials for the temporary connection established by the contractor are to be CE mark.

    - Use of the Site

The Contractor shall restrict his activities to within the sites and shall avoid entry on to any other lands except where the Contractor has made his own arrangements for such entry or the owner has arranged for this entry. Any trespass, damage or claims arising from such entry shall be the sole responsibility of the Contractor, who shall hold the Contracting Authority indemnified against all claims arising from such trespass or damage.

    - Weather Conditions

Without limiting his liabilities, the Contractor shall make suitable arrangements to protect the works and the temporary works, against the effects of the weather.

    - Protection of Existing Works and Services

The Contractor shall explain himself with the position of all existing services such as sewers, surface water drains, cables for electricity and telephone, telephone and lighting, poles, water mains, heating supply pipes and the like, before commencing any excavation or other works likely to affect the existing services. The Contractor will be held liable for all damages to road, main pipes, electrical cables, lines or services of any kind caused by him or his Sub-Contractors in the execution of the works. The Contractor shall make good any damage without delay and, if necessary, carry out any further work ordered by the Supervisor. The Contractor shall indemnify the Contracting Authority against any claims in this respect. In all case where such works or services are exposed, they shall be properly shored, hung up or otherwise protected. Special care shall be exercised in filling and compacting the ground under mains, cables, etc., and not to cover up any water meters, stopcock boxes and similar items. Installations adjacent to the works shall be kept securely in place until the work is completed and shall then be made as safe and permanent as before.

    - Temporary Buildings for Use by the Contractor

The Contractor shall provide and maintain in a perfectly usable and watertight condition, on land in possession of the beneficiary or such lands as he may acquire for the purposes of the works with the approval of the Supervisor, such temporary buildings of timber, galvanized iron, or other suitable material as may be necessary for his general use in connection with the works. As well as buildings for the temporary accommodation of any labor, and for the use of the persons employed by him. Before constructing such buildings he shall supply to the Supervisor a plan or plans showing their positions and nature which shall be in all respects to the approval of the Supervisor. Prior to preparing his Tender, the Contractor shall satisfy himself as to what will be required especially with regard to labor accommodation. The cost of providing temporary buildings for use by the Contractor shall be spread over the works and included in the Breakdown of Prices for those works.

    - Health, Safety and Accidents

The Contractor is obliged to observe all the stipulated measures pertaining to fire protection, protection at work as well as hygienic and technical conditions such as Fire Protection Regulations Rule Book on general Measures and Normative of Protection at work concerning building facilities intended for the work and other subsidiary premises. The Contractor shall ensure, so far as is reasonably practicable and to the satisfaction of the Contracting Authority, the health, safety and welfare at work of his employees including those of his Sub-Contractors and of all other persons on the Site.

His responsibilities shall include: the provision and maintenance of equipment and systems of work must be safe and without risks to health; the execution of suitable arrangements for ensuring safety and absence of risks to health in connection with the use, handling, storage and transport of articles and substances; the provision of protective clothing and equipment, first aid stations with such personnel and equipment as are necessary and such information, instruction, training and supervision as are necessary to ensure the health and safety at work of all persons employed on the works all in accordance with Laws and all local By- Laws; designation as Safety Officer of one of his senior staff who shall have specific knowledge of safety regulations, and experience of safety precautions on similar works and who shall advise on all matters affecting the safety of workman and on measures to be taken to promote such safety; the provision and maintenance of access to all places one the site in a condition that is safe and without risk of injury; the provision of adequate water-borne sanitation, refuse collection and disposal, complying with the Laws and all local By-Laws and to the satisfaction of the Supervisor, for all site offices, workshops erected on the camp site; the provision of suitable latrines and other sanitary arrangements at the site where work is in progress to the satisfaction of the Medical Officer in the area and of the Supervisor; the execution of appropriate measures in consultation with the appropriate Public Health Authority to control within the site; reporting details of any accident to the Supervisor as soon as possible after its occurrence; the provision and maintenance of adequately equipped first aid station on the site of the works.

### A short guide to the personal protective equipment at work (PPE)

What is PPE?

PPE is defined in Regulations as ‘all equipment (including clothing affording protection against the weather) which is intended to be worn or held by a person at work and which protects him against one or more risks to his health or safety’, e.g. safety helmets, gloves, eye protection, high-visibility clothing, safety footwear and safety harnesses..

What do the Regulations require?

The main requirement of the PPE at Work Regulations is that personal protective equipment is to be supplied and used at work wherever there are risks to health and safety that cannot be adequately controlled in other ways. The Regulations also require that PPE:

• is properly assessed before use to ensure it is suitable;

• is maintained and stored properly;

• is provided with instructions on how to use it safely; and

• is used correctly by employees.

Assessing suitable PPE

To allow the right type of PPE to be chosen, carefully consider the different hazards in the workplace. This will enable you to assess which types of PPE are suitable to protect against the hazard and for the job to be done. Ask your supplier for advice on the different types of PPE available and how suitable they are for different tasks. It may be necessary in a few particularly difficult cases to obtain advice from specialist sources and from the PPE manufacturer. Consider the following when assessing whether PPE is suitable:

• Is it appropriate for the risks involved and the conditions at the place where exposure to the risk may occur? For example, eye protection designed for providing protection against agricultural pesticides will not offer adequate face protection for someone using an angle grinder to cut steel or stone.

• Does it prevent or adequately control the risks involved without increasing the overall level of risk?

• Can it be adjusted to fit the wearer correctly?

• Has the state of health of those who will be wearing it been taken into account?

• What are the needs of the job and the demands it places on the wearer? For example, the length of time the PPE needs to be worn, the physical effort required to do the job and the requirements for visibility and communication.

• If more than one item of PPE is being worn, are they compatible? For example, does a particular type of respirator make it difficult to get eye protection to fit properly?

The hazards and types of PPE

Eyes

Hazards: chemical or metal splash, dust, projectiles, gas and vapors, radiation.

Options: safety spectacles, goggles, face shields, visors.

Head

Hazards: impact from falling or flying objects, risk of head bumping, hair entanglement.

Options: a range of helmets and bump caps.

Breathing

Hazards: dust, vapor, gas, oxygen-deficient atmospheres.

Options: disposable filtering face piece or respirator, half- or full-face respirators, air-fed helmets, breathing apparatus.

Protecting the body

Hazards: temperature extremes, adverse weather, chemical or metal splash, spray from pressure leaks or spray guns, impact or penetration, contaminated dust, excessive wear or entanglement of own clothing. Options: conventional or disposable overalls, boiler suits, specialist protective clothing, e.g. chain-mail aprons, high-visibility clothing.

Hands and arms

Hazards: abrasion, temperature extremes, cuts and punctures, impact, chemicals, electric shock, skin infection, disease or contamination.

Options: gloves, gauntlets, mitts, wrist cuffs, armlets.

Feet and legs

Hazards: wet, electrostatic build-up, slipping, cuts and punctures, falling objects, metal and chemical splash, abrasion.

Options: safety boots and shoes with protective toe caps and penetration-resistant mid-sole, gaiters, leggings, spat.

Training

• Make sure anyone using PPE is aware of why it is needed, when it is to be used, repaired or replaced and its limitations.

• Train and instruct people how to use it properly and make sure they are doing this.

• Because PPE is the last resort after other methods of protection have been considered, it is important that users wear it all the time they are exposed to the risk. Never allow exemptions for those jobs which take ‘just a few minutes’.

• Check regularly that PPE is being used and investigate fully any reasons why it is not. Safety signs can be useful reminders to wear PPE

Maintenance

Make sure equipment is:

• well looked after and properly stored when it is not being used, for example in a dry, clean cupboard, or in the case of smaller items, such as eye protection, in a box or case;

• kept clean and in good repair - follow the manufacturer’s maintenance schedule (including recommended replacement periods and shelf lives). Simple maintenance can be carried out by the trained wearer, but more intricate repairs should only be done by specialists. Make sure suitable replacement PPE is always readily available.

# DESCRIPTION OF MATERIALS AND WORKS

## Dismantling of Containerized Blocks in Prishtina

### Dismantling of Container Block #3

Block 3 consists of 35 office containers of dimensions 605x244cm each, 2 ablution units of same dimensions 605x244cm and 2 ablution unit of 244x255cm.

Organised in two floors (G+1) covered with steel structure trusses and pre painted metal sheet positioned over wooden purlins.

Containers are welded among them on the lower and upper beam, see picture below.

Ground floor corridor and staircase are painted concrete while 1st floor corridor is of metal structure with PVC layer, plywood, insulation and plastic sheet.

Building has fire exists with metal external staircase as presented in the drawings.

The building is to be dismantled completely and only the strip foundation is to remain as is

(not to be demolished).

Any Barcoded item/asset such as containers itself, rack cabinets, AC Units, boilers are to be handed over to EULEX Representatives while all the debris to be removed from site to a designated dump site.

Containers once dismantled, are to be made free of all the wall panels leaving only the roof structure installed, floor layers are to be dismantled as well (PVC layer, plywood, insulation, plastic sheet, metal sheet).

Structure is to be preserved for reuse. Dismantling is to be done with due care, so the steel structure of containers is preserved, concrete corridor of the ground floor is to be demolished as required to release the beams/structure of containers and prevent any damage.

Power supply: There is an electrical panel attached to the building which is to remain in the same position as other buildings are connected to the same. Contractor is to prepare foundation / supporting structure for the same and carry the dismantling of container at the spot with due care. All other electrical installations in the building are to be dismantled and disposed.

Plumbing: EULEX Plumber will disconnect the building from the main networks (water and sewerage). Contractor is to carry any interior dismantling works of Block 3 related to sewerage and water from all ablution units including partitions, sanitary elements, etc.

There are two internal hydrants / not active in the building the cabinets will be dismantled and handed over to EULEX Representative.

Fire Alarm: EULEX will disconnect the fire alarm from the main system. Installation/wiring is to be dismantled and disposed by contractor.

Heating System: Building is supplied with central heating system with radiators. Contractor is to disconnect the heating from the main supply line and seal/insulate the connection point on the two main pipes where the building is connected.

Interior installations are to be dismantled and disposed.

CIS: Rack cabinets complete will be withdrawn by EULEX, all other connections/cabling to be dismantled and debris removed from site.

Proper equipment, tools, scaffolding are to be used for dismantling works with specific attention to Health and Safety for the working team and users of the Compound.

See provided dwg 01/01a showing the floor plans of the building.

1 Exterior picture of Block 3 – west and north facade

2.  Ground floor concrete corridor                        3. Electrical Panel

4. Concrete interior staircase                                          5. 1st floor corridor/ceiling

6. Welding positions of containers

### Dismantling of Container Block 4

Block 4 has the same floor plans /layout as Block 3 with some differences on the overall structure.

It consists of 35 office containers of dimensions 605x244cm each, 2 ablution units of same dimensions 605x244cm and 2 ablution unit of 244x255cm.

Organised in two floors (G+1) covered with steel structure trusses and pre painted metal sheet.

Refer to point 4.1.1 for all details and dwg#02/dwg02a.

Different from Block 3 both corridors and staircases (external and internal) are metal structures. Layers of corridors: Laminate, plywood, insulation, plastic sheet.

7 Front facade of Block 4 and corridor

8 Internal corridor & Heating System connection point

### Dismantling of Container Block 5

Block 5 consists of 7 office containers of dimensions 605x244cm each, 2 ablution units of same dimensions 605x244cm and 1 container of dim. 255x244cm.

Organised in ground floor (G+0) covered with steel structure trusses and pre painted metal sheet.

Containers are welded among them on the lower and upper beam similar to block 3 and 4.

The building is to be dismantled completely and only the strip foundation is to remain as is

(not be demolished).

Any Barcoded item/asset such as containers itself, rack cabinets, AC Units, boilers are to be handed over to EULEX Representatives while all the debris to be removed from site to a designated dump site.

Containers once dismantled, are to be made free of all the wall panels leaving only the roof structure installed, floor layers are to be dismantled as well (PVC layer, plywood, insulation, plastic sheet, metal sheet).

Structure is to be preserved for reuse. Dismantling is to be done with due care, so the steel structure of containers is preserved.

Power supply: All electrical installations in the building are to be dismantled and disposed. EULEX Electrician will assist to disconnect the building from the main power line.

Plumbing: EULEX Plumber will disconnect the building from the main networks (water and sewerage). Contractor is to carry any interior dismantling works of Block 5 related to sewerage and water from all ablution units including partitions, sanitary, etc.

Fire Alarm: EULEX will disconnect the fire alarm from the main system. Installation/wiring is to be dismantled and disposed by contractor.

Heating System: Contractor is to disconnect the heating from the main supply line and seal the connection point on the two main pipes where the building is connected.

Interior installations are to be dismantled and disposed.

CIS: Rack cabinets complete will be withdrawn by EULEX, all other connections/cabling to be dismantled and debris removed from site.

The building has also a metal structure corridor and shed in the front, see dwg # 03 provided which are to be dismantled as well and an extension to the containers as indicated in dwg.

Proper equipment, tools, scaffolding are to be used for dismantling works with specific attention to Health and Safety for the working team and users of the Compound.

See provided dwg 03 showing the floor plans of the building.

9 Block 5 – front facade /west

10   Heating pipes to be disconnected from the main line and main pipes sealed/insulated

### Dismantling of partition walls and floor layers (standalone containers in ESC)

There are other containers located within the EULEX Compound in Prishtina, standalone, second-hand containers which are to be prepared for further use.

Works include dismantling of wall panels and floor layers from the same.

20’ container of dimensions 244x605cm mainly have five panels on the longer side wall and two on the short side wall.

All panels dismantled are to be considered as debris and removed from site to a designated authorised dump site.

Apart from the walls also the floor layers (PVC, plywood, insulation, plastic sheet, and metal sheet) are to be dismantled preserving the structure of containers for further use.

Same as with wall panels all debris from floor dismantling works is to be removed from site.

EULEX reserves the right to keep any door or window panel, or other item before debris is removed. No debris is to leave the site without prior approval of representative of Contracting Authority.

11 Standalone containers in ESC

### Refurbishment of steel structure for all office and ablution containers

For all the containers after dismantling (those of block 3,4,5 and of standalone containers in ESC) the following works are to be carried to prepare the steel structure for further use.

The steel structure is to be thoroughly cleaned by removing any rust or loose materials.

Use wire brushes, sanding or grinders to clean off loose rust.

Make sure all weak, flaky, or loose steel is removed.

Apply Anti-Corrosion Primer through the whole structure.

Make sure all the connection points of container structure elements are tight (bolts/washers).

Notify representative of Contracting Authority if any major damage to the structure is noticed.

Note: the roof panels are to remain in place to keep the structure more firm due to transportation.

Floor: Clean and treat the floor steel structure the same way as the exterior main steel structure.

Apart from the main structure beams of the container the flooring of the containers is divided in 60cm segments with steel profiles running on the short side.

Provide and install galvanized metal sheet on the lowest level of the container as support for new layers to be installed at a later stage.

Galvanized metal sheet th= 0.8 mm bended in corners or supported by L flushing and fixed to the floor profiles.

### Transportation of containers.

Once containers structures as described above are ready, the same are to be transported from Prishtina Compound to the new location in Mitrovica, estimated distance 36 KM.

Due care is to be taken during transportation for safe loading, unloading and transportation of the same with use of proper equipments and machinery.

When loading and unloading attention is to be paid to the point of lifting whether through forklift pockets or corner castings dedicated for lifting to not damage the structure.

The container is to be secured in the truck with heavy-duty ratchet straps, chains or by blocking and bracing.

Ensure the container cannot slide or shift during transport.

All containers are to be safely stored on site in Mitrovica as per instructions of Representative of Contracting Authority.

## Dismantling of IFM Block

This container block is located on the yard of Institute of Forensic Medicine in Prishtina within QKUK.

The block consists of 8 office containers (dim. 605x244 each), 4 on each side with steel structure corridor in between,

Container block is organized as ground floor covered with steel structure roof and pre painted metal sheet.

The containers have been installed recently, and the contractor is to carry the dismantling with due care, taking care that containers are not damaged.

Containers are to be transported to Mitrovica new location as well.

Electricity: EULEX will make the required arrangements with local counterpart to disconnect the block from the power supply connection to the building and will cover this part with our electrician.

It will be responsibility of contractor to carry any work from building panel and further.

There are no plumbing installations.

AC Units and heaters are to be handed over to EULEX Representative and containers to be transported to Mitrovica new location.

No dismantling of walls or floor is to be done for these containers, nor of installation inside the containers but only corridor.

Debris to be cleared from site.

Containers are placed over concrete blocks which are to be handed over to EULEX Representative.

Refer to dwg# 04

## Dismantling of Container Blocks in Mitrovica

Container blocks to be dismantled in Mitrovica are separated in two groups, those that building envelope is with Styrofoam facade ‘Demit Facade” placed over the panels/structure and those similar to the blocks in Prishtina with no additional insulation.

Same as in Prishtina, no foundation is to be demolished, all are to remain as they are.

Container blocks with “Demit Facade” are:

### Dismantling of Reception #01

Reception block consists of 4 office containers ( dim 605x244cm) organised in ground floor as per dwg #05 and picture provided.

Facade/insulation is to be cut at the connection point of containers and removed from the facade panels/structure releasing the containers for dismantling.

Containers are welded among them on the lower and upper beam.

Any Barcoded item/asset such as containers itself, rack cabinets, AC Units, boilers are to be handed over to EULEX Representatives while all the debris to be removed from site to a designated dump site.

Containers once dismantled, are to be made free of all the wall panels leaving only the roof structure installed, floor layers are to be dismantled as well (PVC layer, plywood, insulation, plastic sheet, metal sheet).

Structure is to be preserved for reuse. Dismantling is to be done with due care, so the steel structure of containers is preserved.

Power supply: All electrical installations in the building are to be dismantled and disposed. EULEX Electrician will assist to disconnect the building from the main power line.

Plumbing: No plumbing installations at the Reception.

Fire Alarm: EULEX will disconnect the fire alarm from the main system. Installation/wiring is to be dismantled and disposed by contractor.

CIS: Rack cabinets complete will be withdrawn by EULEX, all other connections/cabling to be dismantled and debris removed from site.

Proper equipment, tools, scaffolding are to be used for dismantling works with specific attention to health and Safety for the working team and users of the Compound.

See provided dwg 05 showing the floor plans of the building.

12 Reception Block #01

### Dismantling of Block # 02

Security Block #02 consists of 7 office containers (605x244cm) and one ablution unit of the same size.

Refer to point 4.3.1 for details.

See dwg#06 and picture below.

Different from reception block the building has an ablution unit therefore has plumbing installations and connections.

Plumbing: EULEX Plumber will disconnect the building from the main networks (water and sewerage). Contractor is to carry any interior dismantling works of for the block related to sewerage and water (piping, sanitary installations, partitions etc).

13 Security Block #02 – Same with “Demit” facade over containers

### Dismantling of Block K

Block K consists of 10 office containers (605x244cm) and 3 of  10’ ablution unit (244X255cm).

Organised in two floors (G+1) with steel structure corridors on the 1st level, concrete corridor on the ground floor with internal and external steel staircases.

Steel structure roof covered with pre painted metal sheets.

Refer to point 4.3.1 for more details due to general similarities.

See dwg # 07 and picture below.

This block is equipped with central heating system with radiators, EULEX technician will disconnect the premises from the main connection, Contractor is to dismantle all the radiators and piping running in the premises and remove debris from site.

Block has 3 small ablution units.

Plumbing: EULEX Plumber will disconnect the building from the main networks (water and sewerage). Contractor is to carry any interior dismantling works related to sewerage and water from all ablution units including partitions, sanitary, etc.

14. View of Block K

### Dismantling of Infirmary #04

Block #4 Infirmary consists of 11 office containers of dimensions 605x244cm each, 2 ablution units of same dimensions 605x244cm.

Organised in ground floor (G+0) covered with steel structure trusses and pre painted metal sheet.

Containers are welded among them on the lower and upper beam.

The building is to be dismantled completely and only the foundation is to remain as is

(not be demolished).

Any Barcoded item/asset such as containers itself, rack cabinets, AC Units, boilers are to be handed over to EULEX Representatives while all the debris to be removed from site to a designated dump site.

Containers once dismantled, are to be made free of all the wall panels leaving only the roof structure installed, floor layers are to be dismantled as well (PVC layer, plywood, insulation, plastic sheet, metal sheet).

Structure is to be preserved for reuse. Dismantling is to be done with due care, so the steel structure of containers is preserved, considering also the building corridor which is concreted.

Power supply: All electrical installations in the building are to be dismantled and disposed. EULEX Electrician will assist to disconnect the building from the main power line.

Plumbing: EULEX Plumber will disconnect the building from the main networks (water and sewerage). Contractor is to carry any interior dismantling works related to sewerage and water from all ablution units including partitions, sanitary, etc.

Fire Alarm: EULEX will disconnect the fire alarm from the main system. Installation/wiring is to be dismantled and disposed by contractor.

Heating System: This block is equipped with central heating system with radiators, EULEX technician will disconnect the premises from the main connection, Contractor is to dismantle all the radiators and piping running in the premises and remove debris from site.

CIS: Rack cabinets complete will be withdrawn by EULEX, all other connections/cabling to be dismantled and debris removed from site.

Proper equipment, tools, scaffolding are to be used for dismantling works with specific attention to health and Safety for the working team and users of the Compound.

See provided dwg 08 and also picture below.

15 Infirmary view #04

### Refurbishment of Steel Structure for all containers and ablution units

Refer to 4.1.5 for all details.

### Transportation of containers

Once containers structures as described above are ready, the same are to be transported from Mitrovica existing Compound to the new site in Mitrovica, estimated distance 4 KM.

Due care is to be taken during transportation for safe loading, unloading and transportation of the same with use of proper equipments and machinery.

When loading and unloading attention is to be paid to the point of lifting whether through forklift pockets or corner castings dedicated for lifting to not damage the structure.

The container is to be secured in the truck with heavy-duty ratchet straps, chains or by blocking and bracing.

Ensure the container cannot slide or shift during transport.

All containers are to be safely stored on site in Mitrovica as per instructions of Representative of Contracting Authority.