European Union Rule of Law Mission in Kosovo

EULEX Kosovo

Ndertesa Farmed

“Muharrem Fejza” p.n.

Lagja e Spitalit

10000 Pristina, Kosovo

www.eulex-kosovo.eu

- CORRIGENDUM NUMBER 1

- TO THE TENDER DOSSIER

- PUBLICATION REFERENCE: PROC/767/18/ Supply of Perimeter lights for EULEX compounds under Framework Contract no.2 (EuropeAid/139454/IH/SUP/XK)

The following corrections are made to the Tender Dossier

- The following corrections are made to Annex II+ III Technical Specifications – line item no. 1:

- The former text:

| Item Number | Specifications | Specifications Offered (incl. brand/model) | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | Supply of Energy Saving Aluminium 80-84W LED Street Lighting Fixtures for Outdoor, 5000-6500K, REX-R001 Qty: 45.00 EA |  |  |  |

- Shall read now:

| Item Number | Specifications | Specifications Offered (incl. brand/model) | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | Supply of Energy Saving Aluminium 80-84W LED Street Lighting Fixtures for Outdoor, 5000-6500K, REX-R001 or 100 watt Street Lighting Fixtures containing of 100 psc of 1W LED’s for Outdoor, 5000-6500K, REX-R001 Qty: 45.00 EA |  |  |  |

- The former text:

Detailed Product Description

Energy Saving Aluminium 80-84W LED Street Lighting Fixtures for Outdoor, 5000-6500K, REX-R001

Technology parameter:

| LED Quantity: | 84 pcs 1W LED |
| --- | --- |
| Input Voltage: | AC 170-250V 50/60Hz |
| Output Voltage: | DC 53V |
| Power Factor: | >0.95 |
| Power Efficiency: | ≥90% |
| LED Luminous Efficiency: | >90Lm/W |
| Light output ratio: | >90% |
| Color Temperature: | 5000-6500K |
| Color Rendring Index: | >80 |
| Junction Temperature: | <70℃ |
| Working Environment: | Environment temperature -35~40℃, Relative humidity 10%~90% |
| Storage Temperature: | When it doesn't work, the storage temperature is -35 ~ 80 degrees |
| Protection Rating: | IP65 |
| Lifespan: | 50000h |
| Body & Lampshade Material: | Aluminum Alloy + Toughened Glass + ABS |
| Net Weight: | 10.3 KG |
| Size: | L505 x W310 x H75mm |

- Shall read now:

Detailed Product Description

Energy Saving Aluminium 80-84W LED Street Lighting Fixtures for Outdoor, 5000-6500K, REX-R001 or 100 watt Street Lighting Fixtures containing of 100 psc of 1W LED’s for Outdoor, 5000-6500K, REX-R001

Technology parameter:

| LED Quantity: | 84 pcs 1W LED |
| --- | --- |
| Input Voltage: | AC 170-250V 50/60Hz |
| Output Voltage: | DC 53V |
| Power Factor: | >0.95 |
| Power Efficiency: | ≥90% |
| LED Luminous Efficiency: | >90Lm/W |
| Light output ratio: | >90% |
| Color Temperature: | 5000-6500K |
| Color Rendring Index: | >80 |
| Junction Temperature: | <70℃ |
| Working Environment: | Environment temperature -35~40℃, Relative humidity 10%~90% |
| Storage Temperature: | When it doesn't work, the storage temperature is -35 ~ 80 degrees |
| Protection Rating: | IP65 |
| Lifespan: | 50000h |
| Body & Lampshade Material: | Aluminum Alloy + Toughened Glass + ABS |
| Net Weight: | 10.3 KG |
| Size: | L505 x W310 x H75mm |

EoT